# Detailed Design Document: Portable HDMI Streaming Device

Version: 1.0

Date: February 2025

Status: Final

Format: Markdown

## Table of Contents

[Executive Summary](#1-executive-summary)

[System Context](#2-system-context)

[Architecture Overview](#3-architecture-overview)

[Hardware Abstraction Layer Details](#4-hardware-abstraction-layer-hal-details)

[Software Architecture and Design](#5-software-architecture-and-design)

[Interfaces and API Design](#6-interfaces-and-api-design)

[Data Flow and Sequence Diagrams](#7-data-flow-and-sequence-diagrams)

[Hardware Acceleration and Performance](#8-hardware-acceleration-and-performance)

[Security and Privacy](#9-security-and-privacy)

[Compliance and Licensing](#10-compliance-and-licensing)

[Platform and Build Environment](#11-platform-and-build-environment)

[Testing and Validation](#12-testing-and-validation)

[Deployment and Operations](#13-deployment-and-operations)

[Documentation and Deliverables](#14-documentation-and-deliverables)

[Risks and Mitigations](#15-risks-and-mitigations)

[Appendix](#16-appendix)

## 1. Executive Summary

### 1.1 Product Overview

The Portable HDMI Streaming Device is a firmware-driven embedded system that connects to a TV via HDMI. It delivers a home-page launcher for streaming apps (Netflix, Prime Video, Hulu, Disney+), streams content over Wi-Fi, and supports control via remote, HDMI-CEC, and a companion mobile app over Bluetooth.

### 1.2 Goals

| Goal | Description |
| --- | --- |
| **Simplicity** | Plug-and-play HDMI connection; minimal user setup |
| **Unified Control** | Single remote for TV and device; CEC integration |
| **Streaming** | Wi-Fi delivery with TLS; multi-codec support |
| **Extensibility** | HAL and SOA architecture; swappable drivers |
| **Reliability** | Graceful error handling; OTA updates |

### 1.3 Target Hardware Platform

SoC: ARM or RISC-V class; target: Cortex-A series or equivalent

Memory: 256 MB RAM minimum; 512 MB recommended for 4K

Storage: Flash for firmware; persistent config

Peripherals: HDMI (1.4+), Wi-Fi, Bluetooth/BLE, IR receiver

### 1.4 Key Features

| Feature | Description |
| --- | --- |
| **HDMI** | 1080p/4K video and audio passthrough |
| **HDMI-CEC** | Power, volume, navigation control of TV |
| **Wi-Fi** | Client for streaming; TLS for secure delivery |
| **Bluetooth** | A2DP audio; GATT control for mobile app |
| **Codecs** | H.265, AV1, VP9, MPEG-4, ProRes |
| **Containers** | MP4, MOV, MKV |
| **DRM** | HAL hooks for Widevine/PlayReady/FairPlay |

### 1.5 Core Use Cases

Launch apps from home page – Navigate with remote, select app icon, launch streaming session

HDMI-CEC TV control – Power on/off TV, volume, navigation passthrough

Wi-Fi streaming – Connect to network, stream from app content servers

Bluetooth control – Pair mobile app, send navigation and playback commands

Mobile app integration – Companion app for discovery, pairing, control

## 2. System Context

### 2.1 System Context Diagram

flowchart TB
    subgraph External["External Systems"]
        User[User]
        TV[TV / HDMI]
        WiFi[Wi-Fi Network]
        App[Bluetooth Mobile App]
        Services[Streaming Services]
        DRM[DRM / Licensing]
    end

    Device[Streaming Device]

    User -->|Remote / IR| Device
    User -->|Touch| App
    Device -->|HDMI video/audio, CEC| TV
    Device -->|TLS streams| WiFi
    Device <-->|BLE GATT| App
    Device -->|HTTPS / SDK / APK| Services
    Device -->|Key provisioning| DRM

### 2.2 External Actors and Systems

| Actor/System | Interface | Description |
| --- | --- | --- |
| **TV / HDMI** | HDMI Type A, CEC bus | Video/audio output; CEC command exchange |
| **Wi-Fi Network** | 802.11, TLS | Stream delivery, OTA updates |
| **Bluetooth Devices** | BLE GATT, A2DP | Mobile app control; audio sink |
| **Streaming Services** | HTTPS, SDK/APK | Netflix, Prime, Hulu, etc. content delivery |
| **DRM / Licensing** | Vendor APIs | Widevine, PlayReady; ProRes, HEVC licensing |

### 2.3 Interfaces to External Systems

| Interface | Protocol | Notes |
| --- | --- | --- |
| **HDMI** | HDMI 1.4/2.0 spec | Video, audio, EDID; HDCP when required |
| **HDMI-CEC** | CEC 1.4 | User control codes, power, standby |
| **Wi-Fi** | 802.11n/ac, wpa_supplicant | TLS for streaming endpoints |
| **Bluetooth** | BLE 4.x+, GATT | Custom control service UUID |
| **Streaming SDKs** | HTTPS, vendor APIs | App-specific; auth, CDN |
| **DRM** | Vendor-specific | Key request/response, license tokens |

## 3. Architecture Overview

### 3.1 High-Level Architectural Diagram

graph TB
    subgraph Application
        Main[main.cpp]
    end

    subgraph Services["Core Services (SOA)"]
        UI[UI Service]
        App[App Launcher]
        Stream[Streaming Service]
        CEC[HDMI-CEC Service]
        BT[Bluetooth Control]
        Config[Config Service]
        Telemetry[Telemetry Service]
        Update[Update Service]
    end

    subgraph HAL["Hardware Abstraction Layer"]
        Display[Display HAL]
        Input[Input HAL]
        WiFi[Wi-Fi HAL]
        BT_HAL[Bluetooth HAL]
        CEC_HAL[CEC HAL]
        Storage[Storage HAL]
        Power[Power HAL]
        Codec[Codec HAL]
        Container[Container HAL]
        VideoPipe[Video Pipeline HAL]
        DRM[DRM HAL]
    end

    subgraph Drivers["Platform Drivers"]
        Mock[Mock Drivers]
        Platform[Platform Drivers]
    end

    Main --> UI
    Main --> App
    Main --> Stream
    Main --> CEC
    Main --> Config
    Stream --> Codec
    Stream --> Container
    UI --> Display
    UI --> Input
    CEC --> CEC_HAL
    BT --> BT_HAL
    Config --> Storage
    Display --> Mock
    Input --> Mock
    CEC_HAL --> Mock

### 3.2 Layer Definitions

| Layer | Responsibility |
| --- | --- |
| **Application** | Main loop, input routing, service orchestration |
| **Core Services (SOA)** | UI, app launch, streaming, CEC, config, telemetry, update |
| **Hardware Abstraction Layer** | Abstract interfaces; platform-independent APIs |
| **Platform Drivers** | Mock (test) or real hardware implementations |

### 3.3 Security and Compliance Layer

Secure boot – Verified boot chain (platform-dependent)

Code signing – Firmware signed; OTA verification

DRM – Content protection via IDrmHal; key isolation

### 3.4 DRM / Content Protection Layer

IDrmHal – Request keys, release session, content protection check

Hooks – Integrate Widevine, PlayReady, FairPlay

Key isolation – Secure storage; TEE where available

## 4. Hardware Abstraction Layer (HAL) Details

### 4.1 HAL Domains and API Surface

| HAL | Interface | Key Methods | Data Models |
| --- | --- | --- | --- |
| **Display** | IDisplayHal | initialize, setDisplayMode, getFramebuffer, present, clear | ResolutionInfo, framebuffer |
| **Input** | IInputHal | initialize, poll, injectKey, setInputCallback | InputEvent, RemoteKey |
| **Wi-Fi** | IWifiHal | connect, disconnect, getState, resolveHost | WifiState |
| **Bluetooth** | IBluetoothHal | initialize, registerControlService, writeCharacteristic | BluetoothState |
| **HDMI-CEC** | IHdmiCecHal | sendPowerOn, sendStandby, sendUserControl, sendRemoteKey | CecMessage, CecUserControlCode |
| **Storage** | IStorageHal | read, write, exists, remove | Key-value strings |
| **Power** | IPowerHal | sleep, wake, getBatteryLevel | - |
| **Audio** | IAudioHal | setVolume, mute | - |
| **Codec** | ICodecDecoder | initialize, decodeFrame, flush, reset | DecodedFrame, EncodedPacket |
| **Container** | IContainerParser | openContainer, readPacket, seek, getTracks | TrackMetadata, EncodedPacket |
| **Video Pipeline** | IVideoPipeline | submitFrame, setOutputColorSpace, setHdrMetadata | DecodedFrame |
| **DRM** | IDrmHal | requestKeys, releaseSession, isContentProtected | - |

### 4.2 Error Handling Strategy

Result enum – `device::Result::OK`, `ERROR_INVALID_PARAM`, `ERROR_IO`, etc.

No exceptions – APIs return Result; callers check and handle

Recovery – State rollback on failure; pipeline flush/reset on decode errors

### 4.3 State Diagrams

### 4.3.1 Initialization State

stateDiagram-v2
    [*] --> Idle
    Idle --> Initializing : initialize()
    Initializing --> Ready : OK
    Initializing --> Error : failure
    Ready --> [*] : shutdown()
    Error --> [*] : shutdown()

### 4.3.2 Stream Pipeline State

stateDiagram-v2
    [*] --> IDLE
    IDLE --> OPENING : open()
    OPENING --> PAUSED : OK
    OPENING --> ERROR : failure
    PAUSED --> PLAYING : play()
    PLAYING --> PAUSED : pause()
    PAUSED --> SEEKING : seek()
    PLAYING --> SEEKING : seek()
    SEEKING --> PAUSED : done (was paused)
    SEEKING --> PLAYING : done (was playing)
    SEEKING --> ERROR : seek failure
    PAUSED --> IDLE : stop()
    PLAYING --> IDLE : stop()
    ERROR --> IDLE : stop()

### 4.3.3 Power Management

stateDiagram-v2
    [*] --> Active
    Active --> Standby : sleep()
    Standby --> Active : wake() / wake-on-remote

## 5. Software Architecture and Design

### 5.1 Service-Oriented Architecture (SOA)

### 5.1.1 Core Services

| Service | Responsibility | Interface |
| --- | --- | --- |
| **UI Service** | Launcher, home page rendering, navigation | IUiService |
| **App Launcher** | App registry, launch, lifecycle | IAppLauncherService |
| **Streaming Service** | Pipeline orchestration, session management | IStreamingService |
| **HDMI-CEC Service** | Key mapping, CEC command generation | IHdmiCecService |
| **Bluetooth Control** | Mobile app command handling | IBluetoothControlService |
| **Config Service** | Persistent settings | IConfigService |
| **Telemetry** | Logging, metrics | ITelemetryService |
| **Update** | OTA workflow | IUpdateService |

### 5.1.2 Inter-Service Communication

| Mechanism | Usage |
| --- | --- |
| **Event Bus** | APP_LAUNCHED, APP_STOPPED; decoupled publish/subscribe |
| **Synchronous calls** | Direct service-to-service (e.g., main → CEC, UI → launcher) |
| **Callbacks** | Status, telemetry, input events |

### 5.2 Modules and Responsibilities

| Module | Responsibility |
| --- | --- |
| **UI Service** | Focus model, D-pad navigation, accessibility-ready design |
| **App Launcher** | App catalog from config; auth hooks; session lifecycle |
| **Streaming Service** | Pipeline open/play/pause/seek; buffering; error recovery |
| **HDMI/CEC Manager** | RemoteKey → CecUserControlCode; TV state sync |
| **Bluetooth Control** | GATT Command/Status; pairing; control channel protocol |
| **Network/Streaming** | TLS, content delivery, DRM licensing, QoS |
| **Update and Telemetry** | Secure OTA; log collection; metrics |

### 5.3 Data Models

| Model | Description |
| --- | --- |
| **AppMetadata** | id, name, icon_path, package_uri, auth_endpoint |
| **TrackMetadata** | type, track_id, duration, VideoTrackInfo, AudioTrackInfo |
| **Streaming session** | app_id, session_id, state (IDLE/PLAYING/PAUSED) |
| **HdrMetadata** | color primaries, transfer, mastering display, MaxCLL/MaxFALL |

## 6. Interfaces and API Design

### 6.1 Codec/Decoder API

See Codec_Container_API.md . Key interface: `ICodecDecoder` with `decodeFrame`, `flush`, `reset`, `getCapabilities`.

### 6.2 Container/Demuxer API

See Codec_Container_API.md . Key interface: `IContainerParser` with `openContainer`, `readPacket`, `seek`, `getTracks`.

### 6.3 Remote Control API

HDMI-CEC pathway: `IHdmiCecHal::sendRemoteKey(RemoteKey)` → `sendUserControl(CecUserControlCode)`

Bluetooth pathway: GATT Command characteristic; see [Bluetooth_Control_Protocol.md](Bluetooth_Control_Protocol.md)

### 6.4 Error Handling and Logging

| Contract | Description |
| --- | --- |
| **Result codes** | OK, ERROR_*; no exceptions across HAL/service boundary |
| **Telemetry hooks** | setStatusCallback, setTelemetryCallback for pipeline |
| **Logging** | Logger utility; levels: INFO, WARN, ERROR |

## 7. Data Flow and Sequence Diagrams

### 7.1 User Flow: Power On → Home Page → App Launch → Playback

sequenceDiagram
    participant U as User
    participant I as Input HAL
    participant M as Main
    participant UI as UI Service
    participant A as App Launcher
    participant S as Streaming Service
    participant P as Stream Pipeline
    participant CEC as HDMI-CEC

    U->>I: Power on
    M->>UI: render()
    M->>CEC: powerOnTv()
    CEC-->>U: TV on
    U->>I: OK (Select)
    I->>M: Key Event
    M->>UI: select()
    UI->>A: launch(netflix)
    A->>S: startSession(netflix)
    S->>P: open() + play()
    P-->>U: Video on TV

### 7.2 Remote Navigation → CEC Mapping

sequenceDiagram
    participant R as Remote
    participant I as Input HAL
    participant M as Main
    participant CEC as HDMI-CEC Service
    participant H as CEC HAL
    participant T as TV

    R->>I: Volume Up
    I->>M: Key Event
    M->>CEC: sendKeyToTv(VOLUME_UP)
    CEC->>H: sendUserControl(VOLUME_UP)
    H->>T: USER_CONTROL_PRESSED 0x41
    T-->>R: Volume increases

### 7.3 Bluetooth Pairing → Mobile App Control

sequenceDiagram
    participant A as Mobile App
    participant BLE as Bluetooth HAL
    participant BT as Bluetooth Control
    participant M as Main
    participant UI as UI

    A->>BLE: Scan, connect, pair
    A->>BLE: Subscribe to Status
    A->>BLE: Write 0x05 (SELECT)
    BLE->>BT: Command callback
    BT->>M: Route to UI/Streaming
    M->>UI: select()

### 7.4 Streaming Flow: App Selection → Decode → HDMI

sequenceDiagram
    participant S as Streaming Service
    participant P as Stream Pipeline
    participant C as Container Service
    participant D as Codec Service
    participant V as Video Pipeline
    participant H as Display HAL

    S->>P: open(path)
    P->>C: open()
    P->>D: createDecoder(track)
    S->>P: play()
    loop Playback
        P->>C: readPacket()
        C->>P: EncodedPacket
        P->>D: decodeFrame()
        D->>P: DecodedFrame
        P->>V: submitFrame()
        V->>H: present()
    end

## 8. Hardware Acceleration and Performance

### 8.1 Decoding Strategies

| Strategy | Use Case |
| --- | --- |
| **Hardware accelerator** | Preferred for H.265, AV1, VP9; SoC-specific |
| **Software fallback** | When HW unavailable or format unsupported |
| **ICodecDecoder::setHardwareAcceleration** | Toggle per decoder instance |

### 8.2 Video Pipeline

Color spaces: BT.709, BT.2020; HDR metadata passthrough

HDR: HDR10, HLG; SMPTE ST 2086 mastering display

Upscaling: Optional; `setUpscalingEnabled`

### 8.3 Power and Thermal

Sleep/wake: IPowerHal; wake-on-remote via Input HAL

Thermal: Platform-specific; throttling hooks (future)

### 8.4 Latency and Determinism

| Target | Value |
| --- | --- |
| **Input-to-action** | < 200 ms |
| **CEC command** | < 100 ms |
| **Frame rate** | 30/60 Hz |
| **Buffering** | Configurable; underrun recovery |

## 9. Security and Privacy

### 9.1 Secure Boot and Code Signing

Platform-dependent – Verified boot chain when supported

OTA – Signed update images; verification before apply

### 9.2 Secure OTA Update Process

Check for updates (HTTPS)

Download signed image

Verify signature

Apply; rollback on failure

### 9.3 DRM and Key Management

IDrmHal – Abstract key request/release

Key storage – Secure element or TEE when available

### 9.4 Bluetooth Security

Pairing – BLE secure pairing; passkey or numeric comparison

Bonding – Persistent keys

### 9.5 Data Handling

Logging – No PII in logs

Telemetry – Opt-in; anonymized

Config – Local storage; no cloud sync by default

## 10. Compliance and Licensing

### 10.1 Standards

| Standard | Application |
| --- | --- |
| **HDMI** | HDMI Specification |
| **HDMI-CEC** | CEC 1.4 |
| **Bluetooth** | Bluetooth SIG |
| **Wi-Fi** | 802.11, regional regulations |

### 10.2 DRM and Codec Licensing

| Item | Notes |
| --- | --- |
| **ProRes** | Apple license required |
| **HEVC** | Patent pool; platform vendor typically covers |
| **AV1** | Royalty-free |
| **Widevine/PlayReady** | Per-vendor terms |

### 10.3 Accessibility and Localization

Navigation – D-pad; focus visible

Localization – UI strings; config-driven (future)

## 11. Platform and Build Environment

### 11.1 Target Hardware Specifications

| Spec | Minimum | Recommended |
| --- | --- | --- |
| **SoC** | Cortex-A class | Cortex-A53+ |
| **RAM** | 256 MB | 512 MB |
| **Flash** | 64 MB | 128 MB |
| **HDMI** | 1.4 | 2.0 |

### 11.2 Toolchain and Build

| Tool | Version |
| --- | --- |
| **Compiler** | GCC/Clang C++17 |
| **Build** | CMake 3.14+, Make |
| **CI** | GitHub Actions |

### 11.3 Dependency Management

Third-party – Codec libraries (platform); optional SDKs

Internal – Header-only types; HAL and services in-tree

### 11.4 CI and Automated Testing

Build – `make all` or `cmake --build`

Tests – `make test`; 44 tests (HAL, services, integration)

## 12. Testing and Validation

### 12.1 Testing Strategy by Tier

| Tier | Scope | Count |
| --- | --- | --- |
| **Unit** | HAL, mocks | Many |
| **Service** | App Launcher, UI, Config, CEC, Codec, Container | Medium |
| **Integration** | Full flow (app launch, CEC, Bluetooth) | Few |

### 12.2 Test Data

Reference media – Licensed samples for ProRes; synthetic for others

Edge cases – No extension, audio-only, corrupt headers

See Test_Suite_Document.md .

## 13. Deployment and Operations

### 13.1 Release Management

Versioning – Semantic versioning (future)

Git – main branch; tags for releases

### 13.2 OTA Update and Rollback

Workflow – Download → verify → apply

Rollback – A/B slots where supported; fallback to previous image

### 13.3 Field Diagnostics

Log collection – Via telemetry or debug port

Status – Pipeline state, CEC presence, WiFi state

## 14. Documentation and Deliverables

### 14.1 Document Formats

| Format | Use |
| --- | --- |
| **Markdown** | Primary; MkDocs for web site |
| **Mermaid** | Diagrams (architecture, sequence) |

### 14.2 Artifacts

| Artifact | Location |
| --- | --- |
| **System architecture** | Implementation_Document.md, architecture/ |
| **API reference** | api/, Codec_Container_API.md, Bluetooth_Control_Protocol.md |
| **Developer guides** | guides/, development/ |
| **Config guide** | guides/hdmi-setup.md |

## 15. Risks and Mitigations

| Risk | Mitigation |
| --- | --- |
| **ProRes licensing cost** | Optional codec; disable if not licensed |
| **DRM integration complexity** | HAL abstraction; vendor-specific drivers |
| **CEC interoperability** | Test on multiple TV brands; fallback to passthrough |
| **Supply chain** | Multi-SoC HAL; swappable drivers |
| **Security vulnerability** | Secure OTA; signed updates |

### Open Questions

SoC-specific hardware decoder integration

Multi-language UI implementation

## 16. Appendix

### 16.1 Glossary

| Term | Definition |
| --- | --- |
| **HAL** | Hardware Abstraction Layer |
| **SOA** | Service-Oriented Architecture |
| **CEC** | Consumer Electronics Control (HDMI) |
| **GATT** | Generic Attribute Profile (BLE) |
| **DRM** | Digital Rights Management |
| **HDR** | High Dynamic Range |
| **PTS** | Presentation Time Stamp |
| **OTA** | Over-The-Air |

### 16.2 Acronyms

| Acronym | Expansion |
| --- | --- |
| HDMI | High-Definition Multimedia Interface |
| CEC | Consumer Electronics Control |
| BLE | Bluetooth Low Energy |
| A2DP | Advanced Audio Distribution Profile |
| TLS | Transport Layer Security |
| HEVC | High Efficiency Video Coding |
| AV1 | AOMedia Video 1 |
| HDR10 | HDR 10-bit |

### 16.3 Data Dictionaries

See `include/streaming_device/types.hpp`, `include/streaming_device/media_types.hpp`.

### 16.4 Change History

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | Feb 2025 | Initial release |

*Related documents: Implementation_Document.md , Product_Requirements_Document.md , Knowledge_Base.md , Test_Suite_Document.md *